1

APPENDIX D

 CONCENTRATED ANIMAL FEEDING OPERATION (CAFO) REGULATIONS
Definitions
Animal Manure: Poultry, livestock, or other animal excreta or mixture of excreta with feed, bedding, or other materials.

Applicant: An individual, a corporation, a group of individuals, partnership, joint venture, owners, or any other business entity having charge or control of one or more concentrated animal feeding operations.
Concentrated Animal Feeding Operation (CAFO):  A lot or facility that stables or confines and feeds or maintains animals for a total of 45 days or more in any 12-month period and meets the criteria for class sizes as found in Table 1.1.1.
Permit:  A permit required by these regulations unless stated otherwise.

Residence: A home must be occupied six (6) months out of the year and have utility hook ups. 

Setback: The distance required for an operation to be measured from a particular entity.  Setbacks are measured from the outermost point of the feedlot and the lagoon to the structure/use as identified on Table 1.1.3.
Shall:  Shall means that the condition is an enforceable requirement of this permit.

Should:  Should means that the condition is a recommendation.  If violations of the permit occur, the County will evaluate whether the producer implemented the recommendations contained in this permit that may have helped the producer to avoid the violation.

General Requirements.

An adequate supply of healthy livestock, poultry and other animals is essential to the well-being of county citizens and the State of South Dakota. This section applies to any concentrated animal feeding operation (CAFO) constructed or used after the effective date of March 28, 2017.  Any facility shall be sufficiently separated from other land uses so as not to unreasonably interfere with or burden the enjoyment of other neighboring lands, consistent with the policy established under this Ordinance. All concentrated animal feeding operations shall comply with the regulations as outlined herein.
If a Class A or B concentrated animal feeding operation, which has a previously issued county permit, changes ownership; the state will transfer the permit. The county will update the permit after the applicant provides proof of state approval. If a Class C concentrated animal feeding operation, which was a previously issued county permit, changes ownership; the new owner will notify the county so that the permit may be updated. The new owner will be required to abide by the permit requirements and letter of assurances that were issued under the permit application. If the county is not notified of the transfer within sixty (60) days, there will be a $200 penalty fee. If no transfer is completed within sixty (60) days, the new owner will be required to submit a new application for approval.
Classes of Concentrated Animal Feeding Operations:
Concentrated animal feeding operations are hereby classified as Class A, B, C or D concentrated animal feeding operations and are defined by the number of animal as listed in Table 1.1.1.
Number of Animals to Define Classes of Concentrated Animal Feeding Operations

Table 1.1.1
	TYPE OF ANIMAL FEEDING OPERATION
	Class A: ANIMAL NUMBERS  EQUAL TO 
	Class B: ANIMAL NUMBERS  EQUAL TO 
	Class C: ANIMAL NUMBERS   EQUAL TO
	Class D: ANIMAL NUMBERS   EQUAL TO

	          
	 
	 
	        
	

	Dairy Cows (mature-milked or dry) & Buffalo
	 
3,501 to 7,000 


	701 to 3,500
	200 to 700
	199 or less

	Cattle other than mature dairy cows or veal calves*
	 5,001 to 10,000
	1,001 to 5,000
	300 to 1,000
	299 or less

	Swine (weighing more than 55 pounds)
	 12,501 to 25,000
	2,501 to 12,500
	750 to 2,500
	749 or less

	Swine (weighing less than 55 pounds)
	 50,001 to 100,000
	10,001 to 50,000
	3,000 to 10,000
	2,999 or less

	Sheep, Lambs, or Goats
	 50,001 to 100,000
	10,001 to 50,000
	3,000 to 10,000
	2,999 or less

	Turkeys
	 280,001 to 550,000
	55,001 to 280,000
	17,000 to 55,000
	16,999 or less

	Laying hens or broilers, and Pheasants**
	 150,001 to 300,000
	30,001 to 150,000
	9,000 to 30,000
	8,999 or less

	Chickens, other than laying hens***
	 625,001 to 1,250,000
	125,001 to 625,000
	38,000 to 125,000
	37,999 or less

	Laying hens ***
	 410,001 to 820,000
	80,001 to 410,000
	25,000 to 80,000
	24,999 or less

	Ducks **
	 25,001 to 50,000
	5,001 to 25,000
	1,500 to 5,000
	1,499 or less

	Ducks ***
	 150,001 to 300,000
	30,001 to 150,000
	9,000 to 30,000
	8,999 or less

	Geese
	 150,001 to 300,000
	30,001 to 150,000
	9,000 to 30,000
	8,999 or less

	Horses
	 2,501 to 5,000
	501 to 2,500
	150 to 500
	149 or less

	Fish Farms
	 
	 
	 
	

	Farrow to Finish
	
	
	
	


Table 1.1.2 Animal Ratios to Calculate Class for Multiple Species Under Common Ownership
	Cattle other than mature dairy cows or veal calves*
	1

	Dairy cows (mature – milked or dry & buffalo)
	1.4

	Swine (weighting more than 55 pounds)
	0.4

	Swine (weighting less than 55 pounds)
	0.1

	Sheep, Lambs, or Goats
	0.1

	Turkeys
	0.018

	Laying hens or broilers, and Pheasants**
	0.033

	Chickens, other than laying hens***
	0.008

	Laying hens***
	0.0122

	Ducks**
	0.2

	Ducks***
	0.033

	Geese
	0.033

	Horses
	2


	NOTES:
	

	
	

	* Cattle includes, but is not limited to, heifers, steers, bulls, and cow/calf pairs.

** Animal feeding operation uses a liquid manure handling system.

*** Animal feeding operation uses other than a liquid manure handling system.


Concentrated Animal Feeding Operation Permit Requirements

Two (2) or more concentrated animal feeding operations under common ownership are a single concentrated animal feeding operation if they adjoin each other (within one mile) or if they use a common area or system for disposal of manure.  Required setbacks for the two (2) or more concentrated animal feeding operations treated as a single operation shall not be less than the minimum setback required for each operation if said operations were treated as individual operations. (See Table 1.1.2)
Any person who owns, operates, or proposes to own or operate a Class A, B, or C concentrated animal feeding operation as defined in these regulations, shall be required to apply for a special exception permit pursuant to these regulations whenever any of the following occurs:
1. A new concentrated animal feeding operation is proposed where one does not exist.

2. An expansion of a concentrated animal feeding operation is proposed that exceeds the number of animal units allowed by an existing county-issued permit.
3. An expansion in the number of animal units of a concentrated animal feeding operation, without a county-issued permit, that existed prior to March 28th, 2017 which would result in the creation of either a Class A, B or C concentrated animal feeding operation

4. A change in ownership of a Class C concentrated animal feeding operation which does not have a previously issued county-permit.  
5. An existing concentrated animal feeding operation is to be restocked after being idle for five (5) or more years.

6. A signed complaint has been received and/or documented by the County Zoning Officer or South Dakota Department of Environment and Natural Resources and after inspection reveals that the concentrated animal feeding operation is in violation of County or State regulations.
Concentrated Animal Feeding Operation Control Requirements:
1. Required Minimum Setbacks and Separation Distance for New Classes A, B, and C Concentrated Animal Feeding Operations and those Existing Concentrated Animal Feeding Operations without a County issued permit expanding into a Class A, B, and C Concentrated Animal Feeding Operations after (date of adoption of new ordinance). See Table 1.1.3.
Minimum Setbacks 1.1.3
	 
	CLASS A 
	CLASS B
	CLASS C
	CLASS D 

	1.   Established Residences not including owners/ operators
	 2 miles
	3,960 feet 
	2,640 feet
	 

	2.  Churches, Businesses,  & Commercially-zoned Areas
	 2 miles
	5,280 feet 
	5,280 feet
	 

	3.  Incorporated Municipality
	 3 miles
	2.5 miles
	2 miles
	 

	D.  Private wells less than 100 feet deep & public water supplies other than the operator
	1,000 feet 
	1,000 feet
	1,000 feet
	 

	E.  Private wells over 100 feet deep
	 250 feet
	250 feet
	250 feet
	 

	F.  Lakes & streams classified as fisheries as identified   by the State
	500 feet 
	500 feet
	500 feet
	 

	4.  Federal, State, & County road ROW housed
	300 feet 
	300 feet
	300 feet
	 

	H.  Federal, State, & County Road ROW open lot
	300 feet 
	300 feet
	300 feet
	 

	5.  Township road ROW housed
	150 feet 
	150 feet
	150 feet
	 

	J.  Township road ROW open lot
	150 feet 
	150 feet
	150 feet
	 


1. Additional Setback and Separation Distance Requirements for Classes A, B and C Concentrated Animal Feeding Operations. 

a. Considerations to Increase Setbacks and/or Separation Distances

The facility shall be sited not closer than those distances indicated in Table 1.1.3.  These setbacks are minimum standards that may be increased by the Board of Adjustment during the conditional use permit issuance process due to concerns of the size of the operation or circumstances unique to a specific concentrated animal feeding operation permit application.  Setbacks shall be measured from the outermost point of the feedlot and the lagoon to the structure/use as identified on Table 1.1.3.
b. Considerations to Decrease Setbacks and/or Separation Distances
If an applicant wishes to place a concentrated animal feeding operation closer than the separation distances set forth in these regulations, the applicant shall obtain Waivers of Distance from all residential property owners within the separation distance.  If the applicant obtains waivers from all residential owners of property within the separation distance, the Zoning Administrator will file the waivers with the Register of Deeds. If the applicant cannot obtain waivers from all residential owners of property within the separation distance, they may request a hearing from the Board of Adjustment. Some considerations for allowing decreased setbacks and/or separation distances includes, but is not limited to, the use of a vegetative buffer, deep pit manure management, or any additional recommendations for odor control from the county.  The Board of Adjustment may allow an exception from the separation distance only when the applicant obtains waivers from all the residential owners of property within the separation distance.  Any authorized person, business, or governmental entity that is within the separation distance may waive the separation distance through a written instrument to be filed with the County Register of Deeds.  This waiver will accompany the sale and/or transfer of the land.

3. Exemptions to Setback and/or Separation Distance Requirements
a. All Concentrated Animal Feeding Operations (CAFO) in operation prior to (date of adoption of new ordinance), which do not comply with the minimum setback requirements, but continue to operate, and are not expanded in a manner which will result in one of the following examples are exempt from setback/separation distance requirements:



Example 1: Class D CAFO expands to a Class A, B, or C CAFO.

Example 2: Class C CAFO expands to a Class A or B CAFO.



Example 3: Class B CAFO expands to a Class A CAFO.

Example 4: Class A CAFO expands by 15% of the number of animal units 
b. A concentrated animal feeding operation which is expanded or constructed, if the title holder of the land benefiting from the distance separation requirement executes a written waiver with the title holder of the land where the CAFO is located, under such terms and conditions which the parties may negotiate. The written waiver becomes effective only upon the recording of the waiver in the office of the Register of Deeds in the county. The title holder of the land benefiting from the distance separation requirement land is the individual or individuals, business entity, governmental entity, bona-fide religious institution, or educational institution from which separation is required. The waiver shall be binding upon the heirs, successors, and assigns of the title holder and shall pass with the land.

c. A concentrated animal feeding operation which is constructed or expanded closer than the required setback/separation distance from the corporate limits of a city, if the incorporated community approves a written waiver. The written waiver becomes effective only after it’s recorded with the Register of Deeds.

d. A concentrated animal feeding operation which existed prior to the creation of a residence, educational institution, commercial enterprise, bona-fide religious institution, incorporated community, if the residence, educational institution, commercial enterprise or bona-fide institution was constructed or expanded or the boundaries of the incorporated community were expanded, after the date that the animal feeding operation was established. The date that the concentrated animal feeding operation was established is the date on which concentrated animal feeding operation commenced operating. A change in ownership or expansion shall not change the date of operation.
e. It is the intention of the Board of Adjustment in the enforcement of this ordinance that when an operator of an existing Concentrated Animal Feeding Operation applies for a permit to expand to another class level, the standards that apply to the expansion will not be applied to existing structures that were built in compliance with accepted industry standards in existence at the time of the construction of such facilities.
4.  Standards for Special Exception Permits
a. The Board of Adjustment or the County Zoning Officer may request information relating to concentrated animal feeding operations not contained in these regulations.

b. The Board of Adjustment may impose, in addition to the standards and requirements set forth in these regulations, additional conditions which the Board of Adjustment considers necessary to protect the public health, safety and welfare.

c. Special Exception permits for concentrated Animal Feeding Operations shall be in effect only as long as other provisions of the permit are being adhered to.

d. When considering an application, the Board of Adjustment will take into consideration current and past violations relating to concentrated animal feeding operations that the applicant has or had an interest in.
e. A neighboring township that adjoins between two counties will follow the regulations of the county that is most restrictive.
f. Permit applicants will be required to file a letter of assurances as required by the Board of Adjustment.  The letter of assurances will be prepared by the county and the applicant and signed by both the applicant and the Chair of the Board of Adjustment.  The permit for the concentrated animal feeding operation is based upon compliance with the regulations herein, and letter of assurances. Any violation of these regulations or non-compliance with the letter of assurances shall be cause for revoking a permit.  If a violation of these regulations or non-compliance with the letter of assurance occurs, permit holders will be notified by registered mail and a hearing before the Board of Adjustment will be held concerning status of the permit.  The Board of Adjustment shall either revoke the permit or set a time line for compliance.  If compliance is not met, the permit shall be revoked and the permit holder ordered to cease operations.
g. It is declared unlawful for any person, firm, or corporation to violate any of the terms or provisions of the concentrated animal feeding operation ordinances.  Violation of this ordinance shall be a misdemeanor and may be punishable by a fine of up to two hundred ($200) dollars for each and every day that any violator fails to comply with the provisions of this ordinance.
h. The county will follow State regulations for time allowed to complete construction of a new CAFO.
i. If the work described in any temporary CAFO permit has not been 50% completed within two (2) years from the date of issuance thereof, said permit shall expire.  Applicant may request in writing an extension from the Zoning Administrator.  The administrator may grant a one-time, two (2) year extension for a temporary CAFO permit.  If the work has not commenced after the extension, a new application will be required.
5. Information Required for Class A, B, C and D Concentrated Animal Feeding Operation 

a. Owner’s, manager’s, management company’s or similar entities name, address and telephone number.

b. Legal descriptions of site.

c. The number and type of animals to be housed by the proposed concentrated animal feeding operation.

d. Concentrated Animal Feeding Operations shall obtain a State General Permit pertaining to the animal species of the Concentrated Animal Feeding Operation, if required by the State of South Dakota.  A County special exception permit may be approved conditioned upon receiving a State General Permit.
e. Information on ability to meet designated setback requirements, including maps showing measured distances and site plan.
f. Notification / Road Agreement of whomever maintains the access road (township and county and state).
